#### Description

DH988B is a 80 Sec. PWM or DAC voice output with high quality speech ,2 input pin and 2 I/O pin simple speech.20 Sec. Voice Length at 6 KHz sampling and 4 bit hardware compression. Built-in oscillator for system clock without an external resistor. Maximum 64 voice groups. One Group contain several steps(voice section ); Maximum 700 steps for all the Groups. User selectable 8bit / 5bit / 4 bit data compression. Support three mode trigger function (Stand Alone, Serial Trigger Mode, CPU Command Mode ). Easy use development system is for function selection and voice combination . PC download the ROM code by the USB Port .

#### Functions

- MAX voice file : 700

- MAX Groups : 64

- MAX Step : 700

- Signal step mute length : 0.64 s ( 6k sample rate )

- Operating Voltage range: 2.4V ~ 3.6V

- Total Voice Duration : 80” (120K Samples)

- Input pin : 2

- I/O pin : 2

- Voice output : PWM and DAC

- Sequential Key : TG1 -> 32 Groups.

TG2 -> 21 Groups.

TG11,TG12-> 1 Group

- Debounce time : 50 us or 10 ms

- On/Off function : only for TG1 & TG2

- Trigger mode (for all Input pins) :

- Key Stand-alone Trigger Mode :

- Edge/Level

- Hold/Unhold

- Retrigger/Irretrigger

- Serial Trigger Mode :

Combinations of TG1 and TG2 to trigger maximum 64 Voice Group.

- CPU Command Mode :

User command through TG1,TG2,TG11 & TG12 with serial CPU Interface to control the Multi-Playback function.

- Output status (for each Output pin):

- Stand by Status. (High/Low Status)

- Busy Low Active.

- Busy High Active.

- LED Flash at 6 Hz.

- LED Flash at 3 Hz.

- LED Flash at 1.5 Hz.

- LED Flash at 0.75 Hz.

- Play rate level :

DH988B can provide different play rate in one code as follow

| Play rate(K) | Play rate(K) |
| --- | --- |
| 3 | 6 |
| 3.2 | 6.4 |
| 3.31 | 6.85 |
| 3.42 | 7.38 |
| 3.55 | 8 |
| 3.69 | 8.72 |
| 3.84 | 9.6 |
| 4 | 10.66 |
| 4.17 | 12 |
| 4.36 | 13.71 |
| 4.57 | 16 |
| 4.8 | 19.2 |
| 5.05 | 24 |
| 5.33 |  |
| 5.64 |  |

#### DH988B Pad Location

CHIP SIZE: (0, 0) (1734,715) UM

DH988B

#### PIN Descriptions

| PIN NAME | I/O | Function |
| --- | --- | --- |
| TG1 | INPUT | Key Stand-alone Trigger Mode : TG1 is used to trigger the maximum 32 Voice Group one by one sequentially. |
|  |  | Serial Trigger Mode : TG1 is used to trigger the maximum 64 Voice Group. |
|  |  | CPU Command Mode : TG1 is used as Data Input Pin (CS). |
| TG2 | INPUT | Key Stand-alone Trigger Mode : TG2 is used to trigger the maximum 21 Voice Group one by one sequentially. |
|  |  | Serial Trigger Mode : TG2 is used to trigger the maximum 64 Voice Group |
|  |  | CPU Command Mode : TG2 is used as Clock Pin (CLK). |
| TG11 | INPUT | Key Stand-alone Trigger Mode : TG11 is used trigger Only One Group. |
|  |  | CPU Command Mode : TG11 is used as Data Input Pin (DIO1). |
|  | OUTPUT | As Output PIN , Output the BUSY or Flash Signal . |
| TG12 | INPUT | Key Stand-alone Trigger Mode : TG12 is used trigger Only One Group. |
|  |  | CPU Command Mode . TG12 is used as Selected Pin (DIO2). |
|  | OUTPUT | As Output PIN , Output the BUSY or Flash Signal. |
| PWM1 | OUTPUT | As a PWM PIN , can Directly drive the Speaker . |
| PWM2 | OUTPUT | As a PWM PIN , can Directly drive the Speaker . |
|  |  | As a DAC PIN , 8 bit D/A Current Output. |
| VDD | INPUT | Power Supply PIN , @ 2.4~3.6 V Working Voltage. |
| VSS | INPUT | Connected to Ground. |

#### DH988B Application Circuit

| #### DH988B 3V Stand-alone Mode DH988B | DH988B |
| --- | --- |

DH988B 3V Serial Mode DH988B

DH988B

DH988B 3V CPU Mode DH988B DH988B

#### DH988B Trigger Timing

| (a) Trigger Pulse Width < Group Length Option Setting = Edge / Unhold | (b) Trigger Pulse Width < Group Length Option Setting =Edge / Hold |
| --- | --- |
| (c) Trigger Pulse Width < Group Length Option Setting = Level / Unhold | (d) Trigger Pulse Width < Group Length Option Setting = Level / Hold |
| (e) Trigger Pulse Width > Group Length Option Setting = Edge / Unhold | (f) Trigger Pulse Width >Group Length Option Seeting = Edge / Hold |
| (g) Trigger Pulse Width > Group Length Option Setting = Level / Unhold | (h) Trigger Pulse Width > Group Length Option Setting = Level / Hold |

| (i) Option Setting = Retrigger | (j) Option Setting = Irretrigger |
| --- | --- |
| (k) TG1 = Sequential Trigger & From Group1~Group5 |  |

#### DH988B Trigger Voice Combination Example

| Voice File | Description |
| --- | --- |
| Voice File A | Hello ( 1.5”) |
| Voice File B | Good Morning (3”) |
| Voice File C | John (1”) |
| Voice File D | Tom (1”) |
| Voice File E | Mary (1.5”) |

Group1 = Step1 + Step 2

Group 2 = Step3 + Step 4 + Step 5

Group 3 = Step 6 + Step 7

Group1 = Hello John

Group 2= Hello Tom Good Morning

Group 3 = Good Morning Mary

Step1 = Voice File A

Step 2 = Voice File C

Step 3 = Voice File A

Step 4 = Voice File D

Step 5 = Voice File B

Step 6 = Voice File B

Step 7 = Voice File E

Total use 3 Group , 7 Steps

Voice duration= Hello + Good Morning + John + Tom + Mary

= 1.5” + 3” + 1” + 1” + 1.5”

= 8”

Total duration = 8” ( 20”-8” = 32” space are free , can add more Voice File )

#### DH988B Serial Mode Timing

TG1=Edge/Unhold/Retriger

TG2=Reset PIN

#### DH988B CPU Mode Timing

| Command | PIN | HEX | [7] | [6] | [5] | [4] | [3] | [2] | [1] | [0] | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| START | DIO1 | A5h | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | Wake up the chip |
|  | DIO2 | 05h | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  |
| END | DIO1 | 81h | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | Power down the chip into standby status |
|  | DIO2 | 05h | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  |
| RESET | DIO1 | 83h | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | Stop the Playback and wait a new command |
|  | DIO2 | 02h | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  |
| PAUSE | DIO1 | 83h | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | Pause the Playback and hold at the Voice data |
|  | DIO2 | 07h | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  |
| RESUME | DIO1 | 05h | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | Resume Playback from the previous Voice data |
|  | DIO2 | 07h | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  |

Play Address Command is generated by the Tool.

#### DH988B Voice Wave & Output Status

#### DC Electrical Characteristics

| ##### Item | # Symbol | # Min | ##### Typ | # Max | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Operating voltage | VDD | 2.4 | 3.0 | 3.6 | V |  |
| Standby current | Isb |  | 1 | 5 | uA | VDD=3V,no load |
| Operating current | Iop |  | 600 |  | uA | VDD=3V,no load |
| Drive current of Output | Iod |  | 10 |  | mA | VDD=3V VOUT=0.6V |
| Sink current of Output | Ios |  | 30 |  | mA | VDD=3V VOUT=2.4V |
| Drive current of PWM | Iod |  | 200 |  | mA | VDD=3V,VOUT=1.5V |
| Sink current of PWM | Ios |  | 200 |  | mA | VDD=3V,VOUT=1.5V |

#### AC Electrical Characteristics

| Symbol | Characteristic | Rating |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Min. | Typ. | Max. |  |
| tdb | Key Trigger debounce time (long) | 14 |  |  | ms |
| tdb | Key Trigger debounce time (short) | 50 |  |  | us |
| tgr | Key Trigger Pulse width |  |  |  |  |
| tPW | Serial Mode Input Pulse Width | 100 |  |  | us |
| tPI | The interval of two pulse | 100 |  |  | us |
| tBH | Busy signal output hold time | 200 |  |  | us |
| tBD | Busy signal output delay time | 200 |  |  | us |
| tdV | Voice ouput delay time | 200 |  |  | us |
| tCS | Chip Select setup time | 1 |  |  | us |
| tDS | Data In setup time | 1 |  |  | us |
| tDH | Data In hold time | 1 |  |  | us |
| tCP | Clock Pulse Width | 1 |  |  | us |
| tCC | Clock Cycle time | 2 |  |  | us |
| tCH | Chip Select hold time | 1 |  |  | us |

#### DH988B Package Information

| DH988B-S08B DH988B-SOP8 | PIN; NAME. 1; TG2. 2; TG1. 3; TG11. 4; TG12. 5; VSS. 6; PWM1. 7; VDD. 8; PWM2/DAC. |
| --- | --- |
| DH988B-D08B DH988B-DIP8 | PIN; NAME. 1; TG2. 2; TG1. 3; TG11. 4; TG12. 5; VSS. 6; PWM1. 7; VDD. 8; PWM2/DAC. |

- Writer Mapping Description

Writer Pin : TG2,TG1,TG11,VSS,PWM2,PWM1,VDD

| OTP Writer Power Board Pin Mapping Table |  |  |  |
| --- | --- | --- | --- |
| DIP 48 | PAD Name | 020-SO8B | 020-D08B |
| 10 - RSTN |  |  |  |
| 11 - GND | 5-VSS | 5-VSS | 5-VSS |
| 12 - SCK | 6-PWM2 | 6-PWM2 | 6-PWM2 |
| 13 - CS | 8-PWM1 | 8-PWM1 | 8-PWM1 |
| 14 - VCC | 7-VDD | 7-VDD | 7-VDD |
| 36 - TEST |  |  |  |
| 37 - VPP | 1-TG2 | 1-TG2 | 1-TG2 |
| 38 – DIO1 | 2-TG1 | 2-TG1 | 2-TG1 |
| 39 – DIO2 | 3-TG11 | 3-TG11 | 3-TG11 |

- Writer Board Slot Location